### Installation

- Insert CD and use File Manager or Explorer to run SETUP.EXE
	4 icons in a group will be created in W31
	4 menu items and an icon in W95+

- Ensure Windows ControlPanel/RegionalSettings/Date is set to d/mm/yyyy (or as appropriate – used when user enters a date)

- Install dongle between PC and printer. (PR41 dongle must be removed)

- Run Prwin, and check that the sample data is present.

- Check the sites screen, a readings screen, and a graphs screen.

- Use FarmMenu/Edit to check the sample data farms.

- Check Help and Agronomy Help

### Upgrade from PR41

- Use FileMenu/Import PR41 to import data from PR41
	Each CONSULT.ALL file will be imported in to a new season. 
	Farms will be created if they do not already exist.

- Repeat for multiple CONSULT.ALL files.

- Import calibrations

- Use Explorer to copy any DWU files used by PR41 from C:\PROBE.

- Use ProbeMenu/Calibrations to check calibration equations.

- Use SitesMenu/Show to see if the sites have been imported.

- Use ReadingsMenu/Show to check readings screen.

- Use GraphsMenu/Show to check graphs.

### New user

- Use ProbeMenu/Calibrations to enter calibration equations.

### Setting up for a new season for all users

- Use FarmMenu/Edit to create any new farms required.

- Use SeasonMenu/New to create a new season.

- Use SitesMenu/New to create new sites.
(Note: Sites are created either from a ‘template’ (with a default of Site 999 in SAMPLE.MDB), or by copying the ‘current’ site.  Make sure that the depth and root zones are correct on the first site before the following sites are created)
(Note: Site Number is the same number used in the field)

### Upload from 503DR

- Check the date with Menu/Clock if it is a 503TDR Hydroprobe

- Check the Serial Number with Menu/SN in the Hydroprobe

- Enter some data into the Hydroprobe, using site numbers that correspond to sites that have already been created.

- Use ProbeMenu/Upload<Settings> to check 503DR model, baud rate, disable printing.

- Use ProbeMenu/Upload to check upload.
(Note: Later uploads can be done with SeasonsScreen<Upload/Post>)

- Check the post file can be edited.

### Post Readings

- Use ProbeMenu/PostReadings to post the file
 (Note: Later use Seasons<Upload/Post>)

- Choose the Post Season (Note: this will be saved for future posts)

- Choose the post file, and check data with <edit>

- Check the post options.

- Check the post date is ‘today’, or correct if using the TDR model

- When finished posting, check on the sites and readings screens that the readings have been posted correctly.

### Groups

- Use ProbeMenu/Groups to check that the ‘latest’ group consists of the sites just uploaded.

- Create some new groups of sites.

### Post Irrigations and Rainfall

- Use ProbeMenu/PostIrrigationsAndRainfall to post gains.

- Check options for automatic calculation of effective gain.

- Post rainfall with user entry to a group of sites.

- Post irrigations with user entry.

- Create an irrigation file, and post irrigations from the file.

- Create an irrigation file from the ‘Latest’ group.

### Backup data

- Use FileMenu/Backup to set the backup file to be the current season MDB file, and set other backup options.

- Use FileMenu/Backup to test backup.
Note:
PRLOCAL.MDB contains Farms, Calibrations, Groups
ALLSEAS.MDB contains the list of seasons
1998DATA.MDB contains all the data for the season 1998

### Registration

- Email or fax the owners name and address details to RSNE so that a registration file can be provided.

| Name: |  |
| --- | --- |
| Company: |  |
| Address: |  |
| Dongle: |  |
| Email: |  |
| Agent: |  |

Research Services New England

8/16 Nicholson Street, Balmain, NSW 2041, Australia

Tel +61 (2) 9810 3563

Fax +61 (2) 9810 3323

Email: support@rsne.com.au

Upgrades: www.rsne.com.au/prwin/upgrades.htm