## Taking measurements with a neutron probe

### Clearing old readings stored in the neutron probe

At the beginning of each day, after the readings has been uploaded and posted, the neutron probe needs formatting to clear old readings.

| Press key | Read |  |
| --- | --- | --- |
| ENTER | READY | wake up the probe |
| FMT | REC 662 | 662 is the number of tubes that can be recorded |
| ENTER | KDATA=0 |  |
| ENTER | DPTHS=8 | confirming 0 keydata items |
| ENTER | SET FMT? | confirming 8 depths |
| ENTER | REC 662 | set format, space for 662 tubes |
| ENTER | READY | back to ‘Ready’ mode |

### Taking readings in the field

Access tubes are installed in groups of three, each tube having the same site number.  The site number should be written on the tube.  The readings from the three tubes will be averaged before posting.

Place the neutron probe on top of the aluminium access tube.  Carefully lower the detector to the last stop on the cable.

| Press key | Read |  |
| --- | --- | --- |
| ENTER | READY | wake up the probe |
| LOG | ID 662 |  |
| 123 | KDATA=0 | enter the site number |
| ENTER | TAKE 8 | and press enter |
| START | COUNT 8 | starting a count... |
|  | M8 15123 | count at this depth = 15123 |
| ENTER |  | press enter lift the cable to the next stop and then press start |
| ENTER | DATA OK? | at the top of the tube... if data is ok press enter |
| ENTER | READY | finished - go to next tube |

These steps assume that you are not entering any keydata items, and that the date and time that the readings were taken will be entered as the readings are posted.  New probes record the date and time automatically.

## Uploading to the PC

After readings have been taken they need uploading to a post file on the PC.

- Connect the probe to the PC with the RS232 cable

- Run Prwin.

- On the Seasons Screen use <Upload/Post>.

The readings will be uploaded and put in a post file called (for example) 12JUN01.PST where ’12 June 2001’ is todays date.

When uploading is finished you will be taken to Post Readings

## Post readings

The readings should already be in the post file.

- Use <edit> to examine the readings to see if there are any errors.
Line 1 is the header line, followed by 16 calibration lines - these lines should not normally need to be changed.
The following lines correspond to each tube.  The first number in the line is the site number, and if that has been wrongly entered in the field it can be changed before posting.  If you do make any changes the file must be saved (FileMenu/Save) before exiting (FileMenu/Exit).

- Make sure that you are posting to the correct season (top centre).

- Check the value of ‘Estimated daily water use’  (dwu)

- Press <Post> The header line from the post file will be displayed.

- If it is ok press <Post>

Prwin will pause at the first site for you to check the date and time.

- Press <Post> and the readings will be averaged and sent to each site.

Pause will pause at each site so that you can change the dwu, and also enter any rainfall or irrigation amounts that have been recorded.

You can now look at the readings or graphs to see the new values.

## Post Irrigations and Rainfalls

Use ProbeMenu/IrrigationsAndRainfalls to post any rainfalls or irrigations that have occurred so that Prwin can keep track of the water balance.

- Choose <UserEntry> (irrigations can also be posted from a file)

- Choose rain, irrigation or flood irrigation

- Enter the amount and the date

- Enter the ‘Estimated daily water use’ (dwu)

- Click <Post>

Gains can be posted to all sites; sites on a farm; or sites in a group.  You can set pause at each site, and then simply skip sites that were not irrigated or did not receive rainfall.

## Viewing the readings

### Sites

Use SitesMenu/Show to see a list of all sites, together with the current state of the site and the predicted irrigation dates.

The list can be sorted (SitesMenu/Sort) by site name, irrigation date, farm number, or any other item.

Use the Farm drop-down at the top of screen to show sites on a single farm.

### Readings

Use ReadingsMenu/Show to see all the readings on a site throughout the season.

### Graphs

Use GraphMenu/Show to see depth, time or both graphs.  Graphs can be zoomed by entering new values on the axes, or using the mouse to drag and zoom.  On the top toolbar you can reset the scales, pan the graph, and move to another site.  On the bottom toolbar you can step through the season with the VCR control, and see the soil moisture values of the current profile in the status windows.

## Create new season and sites

This needs to be done once only at the beginning of each season.

### Use SeasonMenu/New to create a new season

- Enter a season number (default = ‘2000’, where 2000 is the current year) and the name of a data file (default = 2000DATA.MDB)

(In subsequent seasons use <advanced> to copy all the sites, plus full and refill points, from a previous season to save having to re-create the sites)

- Click <Save> and then <Close>

- Choose the new season in the list of seasons and click <Sites>

### Use SitesMenu/New to create the new sites

Each site is an irrigation monitoring unit and will normally have 3 aluminium tubes and needs a different site number.  All other details about a site (depths of readings, root zones, etc) will be copied either from a template site, or from the last site created.

- Click <New> and enter a new site number and name

- Click <New> to create another site, or <Save>, <Close>